Multi-Sensor Deep Learning Fusion for Glacial Lake Tracking and GLOF Hazard Assessment in the Hunza Valley, Gilgit-Baltistan

Deliverable 3 — Paper up to Experiments and Results

Muhammad Ali Saleem, Junaid Minhas, and Safee Akmal

Deep Learning for Perception — Semester Research Project, [FILL: Department, University]

[FILL: email addresses]

Before you submit — delete this box This document contains all chapters up to Experiments and Results, as required for Deliverable 3, with the results of the Colab run (notebooks/glof_pipeline.ipynb; all numbers are also in results/metrics/). Submit it together with the executed notebook. Remaining placeholders: affiliation and e-mails under the title, the GPU model in the training-configuration table, the optional GPU training time, and the venue of the Thati & Ari reference. Delete every blue box before exporting to PDF.

Abstract—Glacial lake outburst floods (GLOFs) are among the most destructive hazards in high-mountain Asia, and the Hunza Valley of Gilgit-Baltistan, Pakistan, has experienced repeated, well-documented events within a single decade. Optical satellite monitoring is interrupted by cloud exactly when lakes change fastest, while synthetic aperture radar (SAR) observes through cloud but is noisier. We present a reproducible Google Earth Engine pipeline that co-registers Sentinel-1 SAR, Sentinel-2 optical and Copernicus DEM data on a common 10 m grid and samples every available acquisition in 90-day pre-event / 45-day post-event windows and seasonal anniversary windows (730 dates) at three sites: the ice-dammed Shishper lake, the surge-dammed Khurdopin lake and Borith Lake as a comparison site next to Passu Glacier. Water labels are generated automatically from Sentinel-2 NDWI with cloud, shadow and snow rules (191 labelled dates). An optical-only U-Net and an early-fusion U-Net (ResNet-18 encoder) trained with optical-channel dropout are compared on held-out years. Under clear conditions both models perform similarly (pixel-pooled IoU 0.649 for fusion versus 0.618 for optical-only), with fusion producing fewer false positives. When all optical inputs are removed to simulate complete cloud cover, the optical-only model fails (IoU 0) while the fusion model retains an IoU of 0.326 (0.440 on Borith Lake, precision 0.83), although it does not recover the small Shishper and Khurdopin lakes from radar alone. Lake-area time series from the fusion model reproduce the drainage of Shishper lake after 29 May 2020 and 7 May 2022 and of Khurdopin lake in July 2017. A transparent equal-weight hazard index reached its highest single value at Shishper 22 days before the 2022 GLOF, but on average ranked the large, stable comparison lake highest in all three pre-event windows, showing that absolute lake area must be normalised per site before it can serve as a hazard signal.

Index Terms—Glacial lake outburst flood (GLOF), Sentinel-1, Sentinel-2, sensor fusion, semantic segmentation, U-Net, time series, hazard assessment, Karakoram

# I. INTRODUCTION

## A. Importance

Glacial lake outburst floods (GLOFs) are sudden releases of water stored behind ice or moraine dams. They can travel tens of kilometres downstream within hours, destroying bridges, roads, farmland and settlements with little or no warning. A global assessment estimated that around 15 million people live within 50 km of a glacial lake and within 1 km of a potential flood channel, with High Mountain Asia the most exposed region [1]. At the same time, glacial lakes are growing rapidly: global glacial-lake volume increased by roughly 48% between 1990 and 2018 [2]. The Hunza Valley in the Karakoram range of Gilgit-Baltistan, Pakistan, is one of the most glacier-active valleys in the world and has experienced repeated, well-documented events within a single decade. The surging Khurdopin Glacier dammed the Shimshal River in 2017; the resulting lake formed in March and drained in late July–early August 2017 [3]. The ice-dammed lake in front of the surging Shishper Glacier drained repeatedly from 2019 onwards [4], and on 7 May 2022 its outburst destroyed the Hassanabad bridge on the Karakoram Highway [5]. These events are recent, documented and recurring, which makes the Hunza basin both a priority for monitoring and an unusually good natural laboratory for testing monitoring methods.

## B. Background

Satellite remote sensing is the only practical way to monitor remote glacial lakes repeatedly. Optical sensors such as Sentinel-2 [6] provide 10 m multispectral imagery every few days, and water is readily separated from land using spectral indices such as the Normalized Difference Water Index (NDWI) [7]. However, optical observation fails under cloud, and cloud is frequent in the Karakoram exactly in the spring and summer months when lakes fill and drain. Synthetic aperture radar (SAR) from Sentinel-1 [8] observes the surface through cloud and in darkness, and calm water appears dark because it reflects the radar pulse away from the sensor; SAR, however, is affected by speckle noise and by layover and shadow in steep terrain. A digital elevation model (DEM) adds terrain context, since lakes form on gentle slopes in valley floors. Deep convolutional networks, in particular the encoder–decoder U-Net [9], have become the standard tool for pixel-wise segmentation of such imagery, and several studies have shown that SAR and optical data carry complementary information about glacial lakes [10], [11], [12]. Cloud platforms such as Google Earth Engine [13] now make multi-sensor archives accessible without manual downloading, which makes dense multi-year time series feasible for a small team.

## C. Problem Statement

Despite this progress, there is no SAR + optical + DEM deep-learning pipeline that tracks Hunza-basin glacial lakes at event-scale temporal density and feeds the tracked extents directly into a hazard score that is checked against more than one documented GLOF. Optical-only monitoring leaves gaps precisely around events, and hazard assessments in the basin still start from static, manually prepared inventories.

## D. Research Gap

The literature reviewed in Section II splits into two strands that rarely meet. Detection studies build increasingly accurate segmentation models — including SAR–optical fusion and foundation models — but almost never check their output against a real outburst. Hazard and susceptibility studies validate against Hunza-basin events but start from lake inventories that were compiled beforehand, rather than from lake extents measured by the model itself [14], [15]. Moreover, the well-documented 2017 Khurdopin drainage [3] has not been used as a machine-learning validation case, and no study evaluates a single pipeline against several events in the same basin.

## E. Evidence

The numbers gathered in this review make the gap concrete. The only fusion-based detection study set in this region reports about 89% detection accuracy but is not tested on Hunza-basin lakes or against a hazard outcome [12]. A side-by-side sensor comparison in Bhutan found that SAR alone reached an F1-score of 0.74 against 0.91 for high-resolution optical imagery on the same lakes [11], which is exactly the motivation for fusing rather than replacing optical data. The Hunza-basin susceptibility study by Muneeb et al. [14] reports high scores for Shishper (0.94) and Passu (0.69) but uses a static inventory, and the Shishper 2022 risk assessment of Ullah et al. [5] reproduces the measured flood but relies on field data rather than an automated pipeline. Foundation-model approaches report the highest segmentation scores (IoU up to 0.96) but have not been run on Hunza-basin lakes [16], [17].

## F. Aims and Objectives

This project aims to close the gap at a scale that fits one semester and a three-person team, with one site owned end-to-end by each member. The specific objectives are to:

- build a reproducible pipeline that acquires and co-registers Sentinel-1, Sentinel-2 and Copernicus DEM data over three Hunza-basin sites for every available acquisition in event-centred and seasonal windows;

- train and compare an optical-only U-Net baseline with an early-fusion SAR + optical + DEM U-Net, including under simulated complete cloud cover;

- convert the fusion model's per-date lake masks — including radar-only dates — into lake-area and growth-rate time series;

- combine these measurements with terrain slope into a transparent hazard index; and

- test retrospectively whether the index rises before the documented Khurdopin (2017) and Shishper (2020, 2022) events while remaining lower at the comparison site over the same periods.

The objectives correspond to three research questions. RQ1: Does SAR + optical + DEM fusion change segmentation accuracy (IoU, F1) relative to an optical-only baseline, and how robust is each model when optical data are unavailable? RQ2: What lake-area and growth-rate patterns are observed at the three sites, and how do they behave around each documented event? RQ3: Does the hazard index behave differently before the documented events than at the comparison site over the same periods? These are research questions, not predetermined conclusions; the retrospective analysis is not an operational early-warning system.

# II. LITERATURE REVIEW

We reviewed nineteen studies published between 2018 and 2026; fifteen of them appeared in 2022 or later. Table I, Table II and Table III summarise, for each study, the method, dataset and region, evaluation metrics, reported performance and main limitation. The studies are grouped into rule-based and single-sensor detection (Table I), fusion and foundation-model detection (Table II), and inventory, hazard and event-documentation studies (Table III).

TABLE I

LITERATURE SURVEY (PART I): RULE-BASED AND SINGLE-SENSOR DETECTION STUDIES

| Ref. (Author) | Year | Method | Dataset / Region | Metrics | Performance | Key limitation(s) |
| --- | --- | --- | --- | --- | --- | --- |
| Wangchuk & Bolch [10] | 2020 | Random Forest, rule-based (GLakeMap) | S1 + S2 + DEM; 8 alpine sites (HMA, Alps, Andes) | Per-site validation | High agreement with reference outlines; no single global accuracy | Thresholds need site-specific tuning; not a learned deep model |
| Kaushik et al. (GLNet) [18] | 2022 | Deep CNN | Multispectral + thermal + microwave + DEM | Accuracy | 0.98 | Needs many auxiliary layers; transferability not independently confirmed |
| Thati & Ari [19] | 2024 | U-Net | Landsat-8; Chandra-Bhaga basin (107 lakes) | P, R, F1 | P 95.98%, R 95.23%, F1 94.23% | Single optical sensor; small basin; no SAR |
| Yin et al. [20] | 2024 | Improved YOLOv5-Seg (attention) | Sentinel-2; High Mountain Asia (23,108 lakes) | P, R, mAP@0.5, F1 | P 0.95, R 0.93, mAP 0.96, F1 0.94 | Optical-only, cloud-sensitive; not evaluated in Karakoram |
| Zhang et al. [21] | 2026 | U-Net + interference-mask framework | Optical (Landsat/S2-class); Hindu Kush-Karakoram-Himalaya, 2000–2022 | Change statistics | 325 additional lakes; area +72.58 ± 6.20 km² | Segmentation accuracy not disclosed; optical-only |
| Adhikari & Regmi [22] | 2025 | EfficientNet-B3, time-series segmentation | Sentinel-1 only (5-yr archive); 4 Nepal lakes | Acc, F1, IoU, P, R | Acc 0.9958, F1 0.9538, IoU 0.9130 | SAR-only; 4 lakes; preprint |

TABLE II

LITERATURE SURVEY (PART II): FUSION, MULTISOURCE AND FOUNDATION-MODEL DETECTION STUDIES

| Ref. (Author) | Year | Method | Dataset / Region | Metrics | Performance | Key limitation(s) |
| --- | --- | --- | --- | --- | --- | --- |
| Ali et al. [12] | 2025 | U-Net, input-level SAR + optical fusion | S1 + S2 + PlanetScope; Karakoram & Hindu Kush | Detection accuracy | ≈ 89% | Few Karakoram training lakes; not tested in Hunza basin |
| Mustafa et al. [23] | 2024 | Machine learning, multisource | S1 + S2 + DEM; Passu/Batura glaciers, Hunza | Not disclosed | Not disclosed at abstract level | Accuracy unavailable; one glacier; no hazard score |
| Pathak et al. [24] | 2025 | Random Forest | S1 + S2 + SRTM DEM (+ PlanetScope); Himachal Pradesh | Classification accuracy | 93.69% (S1+S2+DEM); 94.44% with Planet | Confuses glaciers with supraglacial lakes; classical ML |
| Xu et al. [11] | 2024 | DeepLabv3+ per sensor (not fused) | S1, S2, PlanetScope, Landsat-8; Bhutan (2,563 lakes) | F1 | F1 0.74 (S1) to 0.91 (Planet RGB) | Sensors tested separately, not a fusion model |
| Jiang et al. [16] | 2025 | Prithvi-based U-ViT geo-foundation model | S1 + S2 fusion; North Pakistan (NPK) dataset | F1 | 0.894 (vs < 0.80 for CNN baselines) | Heavy compute; two regional datasets only |
| Abdrash (CryoSentinel) [17] | 2026 | TerraMind foundation model + UperNet | S1 + S2 + DEM; Central Asia (Tien Shan) | IoU | Val. 0.9557; held-out spatial 0.9082 | Non-peer-reviewed preprint; not Karakoram |

TABLE III

LITERATURE SURVEY (PART III): INVENTORY, TIME-SERIES, HAZARD AND EVENT-DOCUMENTATION STUDIES

| Ref. (Author) | Year | Method | Dataset / Region | Metrics | Performance | Key limitation(s) |
| --- | --- | --- | --- | --- | --- | --- |
| Shugar et al. [2] | 2020 | Automated Landsat water classification (Google Earth Engine) | Global glacial lakes, 1990–2018 | Area / volume change | Global lake volume ≈ +48% (1990–2018) | Multi-year composites; optical-only; no event-scale tracking |
| Kumar & Vijay [25] | 2026 | Automated inventory from fused Landsat-8 + S1/S2 + Copernicus DEM | High Mountain Asia, region-wide | Positional accuracy | Median displacement 8.12 m (≈ 0.8 S2 pixel) | Geometric accuracy only; no IoU/F1 |
| Rawlins et al. (GLO) [26] | 2026 | Automated deep-learning framework | S1 + S2; Nepal & transboundary catchments, 2017–2024 | F1 | 0.80–0.92 | Annual product; Nepal focus; not Gilgit-Baltistan |
| Muneeb et al. [14] | 2021 | Analytic Hierarchy Process (multi-criteria) | Sentinel-2 + SRTM DEM + expert weights; Hunza River Basin | Qualitative vs observed expansion | Susceptibility: Shishper 0.94, Passu 0.69 | Subjective weights; static inventory; no deep learning |
| Ullah et al. [5] | 2025 | Integrated hazard–vulnerability–risk framework | Field + remote sensing; Shishper event of 7 May 2022 | Measured flow depth/velocity | Reproduces observed flood behaviour | Needs field data; no automated detection |
| Abbas et al. [15] | 2025 | 13 ML / DL / hybrid susceptibility models | Geospatial factors; Central Karakoram National Park | AUC (ROC) | Best AUC 0.83 (GBM); hybrids 0.81 | Relies on a pre-existing lake inventory |
| Steiner et al. [3] | 2018 | Remote sensing (DEM, Landsat, Planet), feature tracking | Khurdopin Glacier, Shimshal; 2017 surge | Measured flow velocities | Documents surge and dammed-lake formation/drainage in 2017 | Descriptive; no segmentation model or hazard score |

## A. Critical Discussion

### 1) Rule-based and classical machine learning:

Rule-based and classical machine-learning approaches — the random-forest GLakeMap of Wangchuk and Bolch [10], the multisource random forest of Pathak et al. [24] and the Passu/Batura study of Mustafa et al. [23] — are interpretable and need little training data, and they were the first to show that Sentinel-1, Sentinel-2 and DEM data are complementary for lakes. Their weakness is reliance on hand-set thresholds and engineered features that must be re-tuned for each site, and they confuse glaciers with supraglacial lakes [24].

### 2) Single-sensor deep learning:

CNN and U-Net models trained on one sensor reach high scores: F1 of about 0.94 on Landsat-8 [19] and Sentinel-2 [20], and 0.95 with SAR alone on four targeted Nepal lakes [22]. Large-scale optical frameworks such as Zhang et al. [21] extend mapping to the whole Hindu Kush-Karakoram-Himalaya. Optical models, however, inherit optical's core weakness — they cannot see through cloud — while SAR-only models are tested on very few lakes. The GLNet model [18] reaches 0.98 accuracy but depends on several auxiliary data layers whose availability limits transfer.

### 3) Multi-sensor fusion:

Genuine fusion remains rare. Ali et al. [12] fuse SAR and optical channels at the input level with a U-Net in the Karakoram and Hindu Kush and are the closest match to this project's method, but they report accuracy only (≈ 89%) and do not test Hunza-basin lakes. Xu et al. [11] evaluate each sensor separately rather than fusing them, yet provide the clearest evidence that SAR alone (F1 0.74) trails optical (F1 0.91), which argues for fusion rather than replacement.

### 4) Foundation models:

Geo-foundation models fuse SAR and optical data through large pretrained transformer backbones and report the strongest numbers in this review: F1 0.894 on the North Pakistan dataset [16] and IoU up to 0.9557 for CryoSentinel [17]. Neither has been run on Hunza-basin lakes, both are computationally heavy for a semester project, and CryoSentinel is an unreviewed preprint validated only in Central Asia.

### 5) Inventories and time series:

Inventory and time-series products document the large-scale trend of lake growth [2] and are becoming automated and multi-sensor [25], [26]. They are typically annual or multi-year snapshots, report geometric or change statistics rather than segmentation metrics, and do not resolve the weeks around an outburst.

### 6) Hazard assessment and event documentation:

On the hazard side, Muneeb et al. [14] and Abbas et al. [15] reach reasonable predictive performance (AUC up to 0.83) but treat the lake inventory as given rather than measured. Ullah et al. [5] go furthest in validating against a real event — the measured 2022 Shishper flood — but start from field data. Steiner et al. [3] document Khurdopin's 2017 event directly; it is a remote-sensing account with no model attached, which is precisely why it has not yet been used as a machine-learning test case.

## B. Research Gap

Three gaps stand out. First, detection and hazard scoring are built and tested separately: fusion-based detection studies [12] do not check their output against a real outburst, and hazard studies [14], [5], [15] start from prepared lake data rather than a model's own tracking. Second, no reviewed study tests a lake-tracking-to-hazard pipeline against more than one confirmed event in the same basin; Khurdopin's 2017 drainage [3] has never been paired with Shishper's outbursts [4], [5] as a multi-event validation set. Third, existing time-series products are annual and optical, so the weeks around an event — often cloud-covered — are not resolved; a comparison site observed over the same periods is also missing.

## C. Problem Statement

There is currently no SAR + optical + DEM fusion pipeline that tracks Hunza-basin glacial lakes at event-scale temporal density and feeds that tracking directly into a hazard score checked against more than one documented GLOF. This project addresses the gap by building one shared fusion model, applying it to every available Sentinel-1 and Sentinel-2 acquisition around the Khurdopin (2017) and Shishper (2020, 2022) events and in the same seasons of other years, and testing whether a simple, transparent hazard index rises before these events while staying lower at a comparison site next to Passu Glacier over the same periods.

# III. METHODOLOGY AND EXPERIMENTAL SETUP

## A. Proposed Methodology

The proposed approach is an early-fusion U-Net [9] with a ResNet-18 encoder [27] that segments glacial-lake water from co-registered Sentinel-1 SAR, Sentinel-2 optical and Copernicus DEM inputs, compared against an identical U-Net that receives optical inputs only. The workflow (Fig. 1) has six stages: (1) acquisition of every available acquisition in event-centred and seasonal windows from Google Earth Engine [13]; (2) co-registration of all modalities on a common 10 m grid; (3) automatic generation of water labels from optical data; (4) training of the optical-only baseline and the fusion model; (5) evaluation of both models on held-out years, under clear conditions and under simulated complete cloud cover; and (6) application of the fusion model to every date — including dates with radar only — to derive lake-area and growth-rate time series and a transparent hazard index that is compared against the documented events. The final objective is therefore threefold: a performance comparison (RQ1), a robustness test when optical data are missing (RQ1), and a retrospective event analysis (RQ2, RQ3).

Fig. 1. Overview of the proposed pipeline. Blue: data acquisition and preparation; orange: labels and models; green: outputs. The optical-only model is used for the comparison only; the fusion model produces the time series and hazard index.

### 1) Study sites:

The study covers three sites in the Hunza basin (Table IV, Fig. 2). Shishper and Khurdopin are the two event sites. For the Passu comparison site we verified on Sentinel-2 imagery that Passu Glacier has no persistent lake at its terminus during 2017–2022; the monitored water body is therefore Borith Lake, the perennial lake immediately south of the Passu Glacier terminus, which had no outburst in the study period. All site centres were located on the imagery because the approximate coordinates in our earlier project plan were 1–7 km off the lakes.

TABLE IV

STUDY SITES

| Site | Lake type | Centre (lat, lon) | Documented event(s) | Role |
| --- | --- | --- | --- | --- |
| Shishper | Ice-dammed (surging glacier blocks a side valley) | 36.358°N, 74.568°E | Repeated drainage from 2019 [4]; 29 May 2020; 7 May 2022 [5] | Event site |
| Khurdopin | Surge-dammed (Shimshal River) | 36.361°N, 75.449°E | Formed Mar 2017; drainage onset ≈ 21 Jul 2017, drained by 3 Aug 2017 [3] | Event site |
| Passu (Borith Lake) | Perennial lake beside Passu Glacier | 36.432°N, 74.862°E | No outburst during 2017–2022 | Comparison site |

Fig. 2. The three 4.48 × 4.48 km study boxes (Sentinel-2 true colour). The cross marks the lake point used to identify the lake when measuring its area.

## B. Model Architecture

Both models share the U-Net architecture shown in Fig. 3. The encoder is a ResNet-18 that reduces the input resolution in five steps (to 1/32) while increasing the number of feature channels to 512; residual connections make the encoder easy to optimise. The decoder upsamples in five steps, concatenating at each step the encoder feature map of the same resolution (skip connection) and applying two 3 × 3 convolutions with batch normalisation and ReLU. A final 1 × 1 convolution and a sigmoid output the per-pixel water probability.

Rationale. U-Net is the standard architecture for pixel-wise segmentation from limited labels, its skip connections preserve the sharp lake shorelines needed for accurate area, and it is the architecture used by the closest regional fusion study [12]. ResNet-18 was chosen over deeper encoders because the labelled set is small (137 training images) and training has to fit a free Colab GPU; with 14.3 M parameters the model trains in minutes. The encoder is initialised with ImageNet weights [28]; for inputs with more than three channels, the pretrained filters of the first convolution are cycled over the additional channels and rescaled (segmentation-models-pytorch default [29]), and all layers are fine-tuned.

Early fusion. In the fusion model the nine channels of Table V are stacked at the input, so the network learns joint optical–radar–terrain features from the first layer. Early fusion is the simplest fusion strategy, keeps the two models architecturally identical (only the first convolution differs), and therefore isolates the effect of the extra inputs. To make the fusion model usable on cloudy and radar-only dates, it is trained with optical-channel dropout: for 30% of training crops all five optical channels are set to zero, forcing the network to also learn to segment water from SAR and terrain alone.

Fig. 3. U-Net with ResNet-18 encoder. Numbers give feature channels and down-sampling factor; C = 5 for the optical-only model and C = 9 for the fusion model.

TABLE V

MODEL INPUT CHANNELS

| # | Channel | Source | Unit / range | Optical | Fusion |
| --- | --- | --- | --- | --- | --- |
| 1 | B2 (blue) | Sentinel-2 | TOA reflectance, 0–1 | ✓ | ✓ |
| 2 | B3 (green) | Sentinel-2 | TOA reflectance, 0–1 | ✓ | ✓ |
| 3 | B4 (red) | Sentinel-2 | TOA reflectance, 0–1 | ✓ | ✓ |
| 4 | B8 (NIR) | Sentinel-2 | TOA reflectance, 0–1 | ✓ | ✓ |
| 5 | NDWI | Derived from B3, B8 | −1 to 1 | ✓ | ✓ |
| 6 | VV backscatter | Sentinel-1 | dB |  | ✓ |
| 7 | VH backscatter | Sentinel-1 | dB |  | ✓ |
| 8 | Elevation | Copernicus DEM | m a.s.l. |  | ✓ |
| 9 | Slope | Derived from DEM | degrees |  | ✓ |

## C. Dataset and Preprocessing

### 1) Data sources:

All data are open and accessed through the Earth Engine Python API (Table VI). Sentinel-2 Level-1C top-of-atmosphere reflectance is used instead of Level-2A surface reflectance because Level-2A coverage over the Karakoram is incomplete in 2017–2018, which includes the Khurdopin event. Sentinel-1 GRD scenes in Earth Engine are already calibrated, terrain-corrected and converted to decibels; we do not repeat these steps. For each site one orbit direction is used throughout (the direction with more scenes: ascending for Shishper, descending for Khurdopin and Passu) so that viewing geometry is constant.

TABLE VI

DATA SOURCES

| Data | Earth Engine collection | Bands used | Resolution | Role |
| --- | --- | --- | --- | --- |
| Sentinel-1 GRD [8] | COPERNICUS/S1_GRD | VV, VH (dB), IW mode | 10 m | Cloud-independent water signal |
| Sentinel-2 L1C [6] | COPERNICUS/S2_HARMONIZED | B2, B3, B4, B8 (TOA reflectance) | 10 m | Optical bands, NDWI, labels |
| S2 cloud probability [30] | COPERNICUS/S2_CLOUD_PROBABILITY | probability (%) | 10 m (from 20 m) | Cloud mask (not a label) |
| Copernicus DEM [31] | COPERNICUS/DEM/GLO30 | Elevation; slope (derived) | 30 m → 10 m | Terrain channels; slope rule; hazard feature |

### 2) Temporal sampling:

Sentinel-2 revisits about every five days and Sentinel-1 every 6–12 days, so daily data do not exist. Instead, every acquisition is taken inside the windows of Table VII: a dense window from 90 days before to 45 days after each documented event, and ±15-day windows around each event's calendar date (7 May, 29 May and 21 July) in every year from 2017 to 2022. The same windows are applied to all three sites, so each site has observations in every event period and in the same season of years without an event. Every Sentinel-2 day whose mosaic covers at least 95% of the site box is downloaded and paired with the nearest Sentinel-1 scene within ±6 days; Sentinel-1 days without a Sentinel-2 acquisition are downloaded as radar-only dates. This yields 730 dates (Fig. 4, Table VIII). Sentinel-1B failed on 23 December 2021 [32], so radar coverage in 2022 is sparser.

TABLE VII

ACQUISITION WINDOWS (IDENTICAL FOR ALL SITES)

| Window | Dates | Days | Purpose |
| --- | --- | --- | --- |
| Khurdopin event | 22 Apr – 4 Sep 2017 | 136 | 90 d before / 45 d after drainage onset (21 Jul 2017) |
| Shishper 2020 event | 29 Feb – 5 Aug 2020 | 159 | Around the 29 May 2020 drainage (merged with anniversary) |
| Shishper 2022 event | 6 Feb – 21 Jun 2022 | 136 | Around the 7 May 2022 GLOF |
| Anniversaries (May) | 22 Apr – 13 Jun, 2018, 2019, 2021 | 3 × 53 | Same season as the May events, no event |
| Anniversaries (July) | 6 Jul – 5 Aug, 2018, 2019, 2021, 2022 | 4 × 31 | Same season as the July event, no event |
| Total | 10 windows | 714 | Applied identically to all three sites |

Fig. 4. Temporal sampling. Top: every acquisition date per site, split into optical (Sentinel-2) and radar-only (Sentinel-1) dates; dashed lines mark the documented events. Bottom: number of dates per site.

### 3) Co-registration:

For each site a 448 × 448 pixel box (4.48 × 4.48 km) centred on the lake point is defined on a 10 m grid in UTM zone 43N (EPSG:32643). All layers are requested from Earth Engine directly on this grid, so the modalities share CRS, transform, resolution and shape by construction. Same-day Sentinel-2 granules and same-pass Sentinel-1 slices are mosaicked. The 30 m DEM is resampled bilinearly (it is therefore not native 10 m terrain); other layers use nearest-neighbour resampling. Slope is computed from the DEM. Arrays are stored as float16 (cloud probability as uint8).

### 4) Cloud masking:

A pixel is treated as cloud when the Sentinel-2 cloud probability [30] is at least 60%. Cloud components smaller than 1,000 pixels (0.1 km²) are discarded because they are mostly false detections on bright rock and snow.

### 5) Automatic label generation:

No source provides lake outlines for the three sites at the event dates, and hand-digitising hundreds of dates was not feasible, so water labels are generated automatically from Sentinel-2 using NDWI [7]:

NDWI = G − NIRG + NIR	(1)

where G and NIR are bands B3 and B8. A pixel is labelled water when NDWI > 0.2, NIR reflectance < 0.12 (excludes snow, ice and cloud), green reflectance > 0.03 (excludes deep shadow) and DEM slope < 20° (excludes steep shadowed cliffs); water components smaller than 50 pixels are removed. Only near-clear images are labelled: at most 10% cloud in the box and at most 40% very bright (snow or haze) pixels. Pixels within 150 m of cloud are marked ignore and excluded from the loss and from scoring. These thresholds were tuned by visual inspection of label overlays after a first version labelled cloud shadows as water. Of 490 optical dates, 191 received labels (Table VIII); Fig. 5 shows examples. Frozen or snow-covered lake surfaces are labelled as land, which is a known limitation of this labelling strategy.

Fig. 5. Examples of automatic labels (blue = water, darkened = cloud buffer, ignored). Shishper lake before the 2022 GLOF, the Khurdopin dammed lake in June 2017, and Borith Lake.

### 6) Input normalisation:

Each channel is standardised with the mean and standard deviation computed on 60 randomly sampled training images. After standardisation, the optical channels are set to zero (the channel mean) wherever the cloud mask is set and on radar-only dates, and missing values (e.g. no Sentinel-1 scene within ±6 days) are set to zero.

### 7) Train / test split:

Neighbouring dates of the same lake are nearly identical, so a random split would leak information. The split is therefore by year: labelled dates from 2019 and 2021 form the test set (54 images) and those from 2017, 2018, 2020 and 2022 form the training set (137 images; Table VIII). No separate validation set is used: hyper-parameters were fixed in advance, the learning-rate schedule is fixed, and no early stopping or model selection is performed, so the test years are never used for any training decision.

TABLE VIII

DATASET COMPOSITION AND YEAR-WISE SPLIT

| Site | Dates (all) | Optical / radar-only | Labelled | Train (2017, 2018, 2020, 2022) | Test (2019, 2021) |
| --- | --- | --- | --- | --- | --- |
| Shishper | 215 | 121 / 94 | 40 | 27 | 13 |
| Khurdopin | 313 | 248 / 65 | 99 | 73 | 26 |
| Passu (Borith) | 202 | 121 / 81 | 52 | 37 | 15 |
| Total | 730 | 490 / 240 | 191 | 137 | 54 |

### 8) Augmentation:

Training uses random 128 × 128 crops from the full site images, with random horizontal and vertical flips. The fusion model additionally uses optical-channel dropout (Section III-B). Rotations by arbitrary angles are not used because SAR geometry depends on the look direction.

## D. Training Configuration

Both models use the configuration in Table IX. The loss combines binary cross-entropy and soft Dice and is computed only over valid (non-ignored) pixels; with p the predicted probability, y the label and m the validity mask:

ℒ = Σ m·BCE(p, y)N + 1 − 2 Σ m·p·y + 1Σ m·p + Σ m·y + 1	(2)

The Dice term counteracts the strong class imbalance (lake water occupies a few percent of each image), while the cross-entropy term provides stable per-pixel gradients. Training stability is supported by ImageNet initialisation, cosine learning-rate annealing, which lowers the step size smoothly towards the end of training, the masked loss, which prevents cloud pixels with unreliable labels from producing gradients, and data augmentation. Optical-channel dropout also acts as a regulariser that prevents the fusion model from relying on NDWI alone.

TABLE IX

TRAINING CONFIGURATION

| Setting | Value |
| --- | --- |
| Framework | PyTorch [33] + segmentation-models-pytorch [29] |
| Architecture | U-Net, ResNet-18 encoder (ImageNet weights [28]), 1 output channel |
| Input channels | 5 (optical-only) / 9 (fusion) |
| Training input | Random 128 × 128 crops of the 448 × 448 site images |
| Batch size | 16 |
| Iterations | 3,000 on GPU (≈ 29 epoch-equivalents of 137 images); 1,500 if run on CPU |
| Optimiser | Adam [34], initial learning rate 1 × 10⁻³, no weight decay |
| LR schedule | Cosine annealing to 0 over all iterations [35] |
| Loss | Masked binary cross-entropy + soft Dice [36] (cloud pixels ignored) |
| Augmentation | Random horizontal and vertical flips (p = 0.5 each) |
| Optical-channel dropout | Fusion only: all 5 optical channels zeroed for 30% of crops |
| Decision threshold | 0.5 on the sigmoid output |
| Random seeds | NumPy and PyTorch seeds fixed to 0 |
| Hardware | Google Colab, NVIDIA T4 GPU (16 GB) [FILL: confirm GPU shown by !nvidia-smi] |

## E. Evaluation Metrics

### 1) Segmentation:

Predictions are evaluated on the valid (non-cloud) pixels of the test images using intersection-over-union (IoU), F1-score (Dice), precision and recall, with TP, FP and FN the numbers of true-positive, false-positive and false-negative water pixels:

IoU = TPTP + FP + FN,   F1 = 2TP2TP + FP + FN	(3)

Precision = TPTP + FP,   Recall = TPTP + FN	(4)

IoU is the standard segmentation metric and directly reflects errors in lake extent, which is the quantity used downstream. F1 is reported for comparability with the literature reviewed in Section II. Precision and recall separate over-detection (e.g. shadows or wet ice classified as water) from under-detection (e.g. turbid or partly frozen lakes). Overall accuracy is not used because it would be dominated by the land class. Many test images contain no lake water (Khurdopin after 2017, Shishper after each drainage); per-image IoU is undefined for them, and averaging it would score a correct "no water" prediction as zero. The main results are therefore pixel-pooled metrics, for which TP, FP and FN are summed over all test pixels of a site (or of all sites) before the metric is computed. In addition, per-image metrics are reported as mean ± standard deviation over the test images that contain water, and the false-positive area is reported for test images without water.

### 2) Robustness to missing optical data:

To test the cloud-robustness motivation directly, both models are also evaluated after setting all optical channels of the test images to zero (simulated complete cloud cover), using the same labels. The optical-only model then receives no information, while the fusion model can use SAR and terrain.

### 3) Time series:

The fusion model is applied to all 730 dates. Lake area Aₜ on date t is the number of predicted water pixels in connected components that touch a 400 m neighbourhood of the lake point, multiplied by the pixel area (100 m²); this excludes rivers and other water in the box. Area is smoothed with a centred rolling median over three observations (Ãₜ), and the growth rate is computed over roughly 30 days:

Gt = Ãt − Ãt′t − t′ × 30	(5)

where t′ is the latest date at least 30 days before t; growth is not computed when t − t′ exceeds 60 days (gaps between windows).

### 4) Hazard index:

Following the multi-criteria idea of Muneeb et al. [14] in a reduced, transparent form, the hazard index combines three features: smoothed lake area, positive growth rate and the mean DEM slope along the lake outline. Each is min–max normalised over all sites and dates to give a, g and s in [0, 1], and the index is their equal-weight mean:

Ht = at + gt + st3	(6)

Equal weights are a modelling assumption, not fitted to the events. For each documented event the index of all three sites is compared over the 90 days before the event date (mean and maximum), and plotted against days relative to the event. This is a retrospective evaluation, not an early-warning system.

# IV. EXPERIMENTS AND RESULTS

The experiments are organised around the research questions (Table X). All experiments use the data, split and configuration of Section III. Both models were trained once on a Google Colab GPU with identical settings, seeds and data; they differ only in their input channels and in the optical-channel dropout of the fusion model.

TABLE X

OVERVIEW OF EXPERIMENTS

| ID | Experiment | Purpose | RQ |
| --- | --- | --- | --- |
| E1 | Data availability and reference series | Characterise the sampled data and the label-derived reference lake areas | RQ2 |
| E2 | Baseline vs fusion (clear conditions) | Compare optical-only and fusion U-Nets on held-out years | RQ1 |
| E3 | Simulated complete cloud cover | Test segmentation when all optical channels are removed | RQ1 |
| E4 | Lake-area time series | Track lake area with the fusion model on all 730 dates | RQ2 |
| E5 | Hazard-index behaviour | Compare the index of event and comparison sites before each event | RQ3 |

## A. E1: Data Availability and Reference Series

Purpose: to characterise the data available around each event and to establish a model-independent reference. The download produced 730 dates: 215 for Shishper (121 optical, 94 radar-only), 313 for Khurdopin (248 optical, 65 radar-only) and 202 for Passu/Borith Lake (121 optical, 81 radar-only) (Fig. 4). Khurdopin has about twice as many optical dates as the other sites, most likely because it lies where adjacent Sentinel-2 orbits overlap. Radar-only dates make up 33% of all dates. Only 191 of 490 optical dates (39%) are clear enough to be labelled, which quantifies how strongly cloud and snow limit optical monitoring in these windows.

Fig. 6 shows the lake area measured directly from the automatic labels (clear dates only). The documented events are already visible (Table XI): the Shishper lake had water on 30 April 2022 and none on 10 May 2022, three days after the 7 May GLOF; it had water on 25 May 2020 and none by 29 June 2020; and the Khurdopin lake was present from late May to 12 June 2017 and gone by 27 July 2017. The Shishper lake also held water in May 2019 and was empty by late July 2019, consistent with its repeated drainage [4]. Borith Lake stayed between 0.075 and 0.121 km² (interquartile range). Label-derived areas underestimate the event lakes because frozen and very turbid parts of the lakes are not labelled as water.

Fig. 6. Reference lake area derived from the automatic labels (clear optical dates only). Dashed red lines mark documented events (Khurdopin 21 Jul 2017; Shishper 29 May 2020 and 7 May 2022).

TABLE XI

LABEL-DERIVED LAKE AREA AROUND DOCUMENTED EVENTS

| Event | Last labelled water before event | First labelled date after event |
| --- | --- | --- |
| Khurdopin drainage, 21 Jul 2017 | 12 Jun 2017: 0.019 km² | 27 Jul 2017: 0 km² |
| Shishper drainage, 29 May 2020 | 25 May 2020: 0.012 km² | 29 Jun 2020: 0 km² |
| Shishper GLOF, 7 May 2022 | 30 Apr 2022: 0.029 km² (max 0.054 km² on 5 Apr) | 10 May 2022: 0 km² |
| Borith Lake (comparison) | Median 0.107 km² (IQR 0.075–0.121), 52 dates | No event |

## B. E2: Optical-only Baseline versus Fusion

Purpose: to answer the first part of RQ1 under identical conditions. Of the 54 held-out images from 2019 and 2021, 31 contain lake water and 23 do not. Table XII reports pixel-pooled metrics over all test images, Table XIII and Fig. 7 the pooled IoU per site, Table XIV per-image metrics on the images that contain water, and Table XV false positives on the images without water.

Under clear conditions the two models perform similarly. Pooled over all test pixels, the fusion model reaches an IoU of 0.649 (F1 0.787) against 0.618 (F1 0.764) for the optical-only model. The difference comes from precision (0.767 versus 0.706): the optical-only model marks more non-lake pixels as water, and on the 23 images without water it produces false positives on 4 images against 1 for the fusion model. The optical-only model has the higher recall (0.831 versus 0.809) and, averaged over the 31 images with water, a slightly higher IoU (0.584 ± 0.279 versus 0.551 ± 0.274); this difference is well within one standard deviation. Per site, the fusion model is better on the two event lakes (Shishper 0.419 versus 0.372; Khurdopin 0.393 versus 0.329) and slightly worse on Borith Lake (0.761 versus 0.788), which both models delineate well.

TABLE XII

PIXEL-POOLED SEGMENTATION RESULTS ON HELD-OUT YEARS (ALL 54 TEST IMAGES)

| Condition | Model | IoU | F1 | Precision | Recall |
| --- | --- | --- | --- | --- | --- |
| Clear | Optical-only (5 ch) | 0.618 | 0.764 | 0.706 | 0.831 |
| Clear | Fusion (9 ch) | 0.649 | 0.787 | 0.767 | 0.809 |
| Simulated cloud | Optical-only (5 ch) | 0.000 | 0.000 | 0.000 | 0.000 |
| Simulated cloud | Fusion (9 ch) | 0.326 | 0.492 | 0.834 | 0.349 |

Best value per condition in bold. Clear = original inputs; simulated cloud = all five optical channels set to zero.

TABLE XIII

PIXEL-POOLED IOU PER SITE

| Site (test images / with water) | Optical IoU | Fusion IoU | Optical IoU, sim. cloud | Fusion IoU, sim. cloud |
| --- | --- | --- | --- | --- |
| Shishper (13 / 8) | 0.372 | 0.419 | 0.000 | 0.000 |
| Khurdopin (26 / 8) | 0.329 | 0.393 | 0.000 | 0.000 |
| Passu / Borith (15 / 15) | 0.788 | 0.761 | 0.000 | 0.440 |
| All sites (54 / 31) | 0.618 | 0.649 | 0.000 | 0.326 |

TABLE XIV

PER-IMAGE RESULTS ON THE 31 TEST IMAGES CONTAINING WATER (MEAN ± SD)

| Condition | Model | IoU | F1 | Precision | Recall |
| --- | --- | --- | --- | --- | --- |
| Clear | Optical-only | 0.584 ± 0.279 | 0.692 ± 0.271 | 0.672 ± 0.297 | 0.768 ± 0.266 |
| Clear | Fusion | 0.551 ± 0.274 | 0.665 ± 0.274 | 0.684 ± 0.265 | 0.704 ± 0.321 |
| Simulated cloud | Optical-only | 0.000 ± 0.000 | 0.000 ± 0.000 | 0.000 ± 0.000 | 0.000 ± 0.000 |
| Simulated cloud | Fusion | 0.210 ± 0.249 | 0.282 ± 0.325 | 0.375 ± 0.437 | 0.237 ± 0.277 |

TABLE XV

FALSE POSITIVES ON THE 23 TEST IMAGES WITHOUT WATER

| Condition | Model | Images without water | Images with any false positive | Mean false-positive area (km²) |
| --- | --- | --- | --- | --- |
| Clear | Optical-only | 23 | 4 | 0.0029 |
| Clear | Fusion | 23 | 1 | < 0.0001 |
| Simulated cloud | Optical-only | 23 | 0 | 0 |
| Simulated cloud | Fusion | 23 | 1 | 0.0001 |

## C. E3: Robustness to Simulated Cloud Cover

Purpose: to answer the second part of RQ1. When all optical channels are removed, the optical-only model predicts no water at all (IoU 0 at every site), whereas the fusion model retains a pooled IoU of 0.326 — 50% of its clear-condition value (Table XII, Fig. 7). Its radar-and-terrain predictions are conservative but reliable: precision stays high (0.834) while recall drops to 0.349, and it adds a false positive on only one of the 23 images without water (Table XV). The robustness is, however, site-dependent (Table XIII): on Borith Lake the fusion model keeps an IoU of 0.440 (precision 0.839, recall 0.480), but it does not recover the smaller Shishper and Khurdopin lakes from radar alone (IoU 0).

Fig. 7. Pixel-pooled IoU per site of the optical-only and fusion models under clear conditions and with all optical channels removed (simulated complete cloud cover).

## D. Qualitative Results

Fig. 8 shows four held-out dates. On Shishper (6 May 2019) the automatic label covers only part of the turquoise lake, while both models detect a larger part of it; the optical-only model also marks several dark streams and shadowed patches elsewhere in the box as water, whereas the fusion output is limited to the lake — the precision difference of Table XII. Neither model recovers the lake without optical inputs. On Khurdopin (26 July 2021) the label contains only a tiny water patch on the river at the image edge, which both models miss; such cases count as false negatives and explain the low Khurdopin scores. On Borith Lake (29 July 2021) both models match the label closely, and with optical inputs removed the fusion model still recovers the lake outline, with two small false positives at the image edge. On the drained Shishper lake (25 July 2019) both models correctly predict no water.

Fig. 8. Qualitative results on held-out dates. Columns: Sentinel-2 image, automatic label, optical-only prediction, fusion prediction, and both models with all optical channels removed (simulated cloud).

## E. E4: Lake-Area Time Series

Purpose: to answer RQ2. Fig. 9 shows the lake area derived by the fusion model for all 730 dates, coloured by data source, and Table XVI summarises the series. At Shishper, lake water appears on clear spring dates before each event — up to about 0.05 km² in April–May 2020 and about 0.10 km² in April 2022 — and the area is zero on every date after 29 May 2020 and after 7 May 2022, apart from one cloudy-date detection of about 0.025 km² in June 2022. The lake also appears in May 2019 and spring 2021 and is gone by mid-summer in both years. At Khurdopin, small areas of 0.01–0.02 km² are detected in May–June 2017 and none after the July 2017 drainage or in any later year. Borith Lake is detected on almost every date, at 0.02–0.15 km², with large scatter: radar-only dates give systematically smaller areas (about 0.04–0.09 km²) than clear optical dates (0.10–0.15 km²), and several winter and early-spring dates give zero when the lake is frozen or snow-covered. At the two event lakes, radar-only dates show zero area even while the lake existed, consistent with E3. A single cloudy date in April 2020 gives a spurious 0.20 km² at Shishper.

Fig. 9. Lake area from the fusion model at the three sites (points coloured by source: optical clear, optical cloudy, radar only; black line: 3-point rolling median). Red dashed lines: events at that site; pink: events at the other sites.

TABLE XVI

SUMMARY OF LAKE-AREA TIME SERIES (FUSION MODEL; AREAS READ FROM THE SERIES IN THE CORRESPONDING FIGURE)

| Site | Dates: optical clear / cloudy / radar-only | Lake area before event (fusion) | After event |
| --- | --- | --- | --- |
| Shishper | 56 / 65 / 94 | ≈ 0.05 km² (Apr–May 2020); ≈ 0.10 km² (Apr 2022) | 0 on all dates after 29 May 2020 and after 7 May 2022 (one cloudy-date detection of ≈ 0.025 km² in Jun 2022) |
| Khurdopin | 127 / 121 / 65 | ≈ 0.01–0.02 km² (May–Jun 2017) | 0 after Jul 2017 and in all later years |
| Passu (Borith) | 67 / 54 / 81 | 0.02–0.15 km² throughout (no event) | — |

## F. E5: Hazard-Index Behaviour

Purpose: to answer RQ3. Fig. 10 plots the hazard index of all three sites against days relative to each event, and Table XVII reports the mean and maximum index over the 90 days before each event. The event site did not have the highest mean index in any of the three windows: Borith Lake scored highest on average before the Shishper 2020 event (0.273 versus 0.146 for Shishper), before the Shishper 2022 GLOF (0.291 versus 0.171) and before the Khurdopin 2017 drainage (0.231 versus 0.092 for Khurdopin). The highest single value of any window, however, was reached by Shishper before the 2022 GLOF (0.737, about 22 days before the event, when the lake grew rapidly), and Shishper's index rose clearly above its baseline in the weeks before both of its events. Khurdopin's index stayed near its baseline (maximum 0.155) because only a small lake area was detected.

Fig. 10. Hazard index of the three sites in the 90 days before and 45 days after each documented event (3-point rolling mean).

TABLE XVII

HAZARD INDEX IN THE 90 DAYS BEFORE EACH EVENT

| Event window (90 d before) | Site | Mean index | Max index | Dates |
| --- | --- | --- | --- | --- |
| Khurdopin 2017 | Khurdopin (event) | 0.092 | 0.155 | 21 |
|  | Shishper | 0.079 | 0.079 | 19 |
|  | Passu (Borith) | 0.231 | 0.339 | 17 |
| Shishper 2020 | Shishper (event) | 0.146 | 0.418 | 29 |
|  | Khurdopin | 0.079 | 0.079 | 43 |
|  | Passu (Borith) | 0.273 | 0.664 | 29 |
| Shishper 2022 | Shishper (event) | 0.171 | 0.737 | 31 |
|  | Khurdopin | 0.079 | 0.079 | 40 |
|  | Passu (Borith) | 0.291 | 0.626 | 23 |

Highest value per event window in bold.

## G. Computational Performance

Table XVIII lists model size and run times. Both models have practically the same size — the fusion model adds only the weights of four extra input channels in the first convolution — so the robustness gained by fusion comes at no meaningful computational cost. On a Colab GPU both models train within a single notebook session; on a 4-thread laptop CPU a training step takes about 2.6 s, which makes CPU training impractical (about 2.2 h for both models).

TABLE XVIII

COMPUTATIONAL COST

| Quantity | Optical-only | Fusion |
| --- | --- | --- |
| Trainable parameters | 14,334,481 | 14,347,025 |
| Training iterations (batch 16) | 3,000 | 3,000 |
| Training time on Colab GPU | [FILL: optional, min] | [FILL: optional, min] |
| Training step, batch 16 (4-thread CPU, measured) | ≈ 2.6 s | ≈ 2.6 s |
| Inference per 448 × 448 image (4-thread CPU, measured) | ≈ 0.7 s | ≈ 0.7 s |
| Data download (Earth Engine, 730 dates, 8 parallel requests) | ≈ 13 min (shared) |  |

Guide — delete this box before submitting Optional: to report GPU training time, wrap the two train(...) calls in notebook section 7 with t0 = time.time() / print(time.time() - t0) and re-run; otherwise delete the "Training time on Colab GPU" row.

## H. Results Summary

Under clear conditions the fusion model achieved the best pixel-pooled segmentation (IoU 0.649, F1 0.787 versus 0.618 and 0.764 for the optical-only baseline), mainly through fewer false positives, while the optical-only model had slightly higher recall and per-image IoU. With all optical inputs removed, the optical-only model failed completely and the fusion model retained an IoU of 0.326 overall and 0.440 on Borith Lake, but did not detect the small event lakes (RQ1). The fusion-model time series reproduced the timing of the Shishper drainages of May 2020 and May 2022 and the Khurdopin drainage of July 2017, with useful radar-only observations only at Borith Lake (RQ2). The hazard index reached its highest single value at Shishper before the 2022 GLOF, but on average it ranked the comparison site highest in all three pre-event windows (RQ3).

# REFERENCES

[1]	C. Taylor, T. R. Robinson, S. Dunning, J. R. Carr, and M. Westoby, "Glacial lake outburst floods threaten millions globally," Nature Communications, vol. 14, art. 487, 2023, doi: 10.1038/s41467-023-36033-x.

[2]	D. H. Shugar et al., "Rapid worldwide growth of glacial lakes since 1990," Nature Climate Change, vol. 10, pp. 939–945, 2020, doi: 10.1038/s41558-020-0855-4.

[3]	J. F. Steiner, P. D. A. Kraaijenbrink, S. G. Jiduc, and W. W. Immerzeel, "Brief communication: The Khurdopin glacier surge revisited – extreme flow velocities and formation of a dammed lake in 2017," The Cryosphere, vol. 12, no. 1, pp. 95–101, 2018, doi: 10.5194/tc-12-95-2018.

[4]	G. Khan, S. Ali, et al., "Expansion of Shishper Glacier lake and recent glacial lake outburst flood (GLOF), Gilgit-Baltistan, Pakistan," Environmental Science and Pollution Research, 2021.

[5]	S. Ullah et al., "An integrated approach for GLOF hazard, vulnerability and risk assessment in the Karakoram Mountain Range of northern Pakistan," Journal of Mountain Science, 2025.

[6]	M. Drusch et al., "Sentinel-2: ESA's optical high-resolution mission for GMES operational services," Remote Sensing of Environment, vol. 120, pp. 25–36, 2012.

[7]	S. K. McFeeters, "The use of the Normalized Difference Water Index (NDWI) in the delineation of open water features," International Journal of Remote Sensing, vol. 17, no. 7, pp. 1425–1432, 1996.

[8]	R. Torres et al., "GMES Sentinel-1 mission," Remote Sensing of Environment, vol. 120, pp. 9–24, 2012.

[9]	O. Ronneberger, P. Fischer, and T. Brox, "U-Net: Convolutional networks for biomedical image segmentation," in Proc. MICCAI, LNCS vol. 9351, 2015, pp. 234–241.

[10]	S. Wangchuk and T. Bolch, "Mapping of glacial lakes using Sentinel-1 and Sentinel-2 data and a random forest classifier: Strengths and challenges," Science of Remote Sensing, vol. 2, art. 100008, 2020.

[11]	X. Xu et al., "Combined use of multi-source satellite imagery and deep learning for automated mapping of glacial lakes in the Bhutan Himalaya," Science of Remote Sensing, vol. 10, art. 100157, 2024.

[12]	S. Ali et al., "Deep learning model to detect glacial lakes using high-resolution optical and radar satellite images," Remote Sensing Applications: Society and Environment, 2025.

[13]	N. Gorelick, M. Hancher, M. Dixon, S. Ilyushchenko, D. Thau, and R. Moore, "Google Earth Engine: Planetary-scale geospatial analysis for everyone," Remote Sensing of Environment, vol. 202, pp. 18–27, 2017.

[14]	F. Muneeb, S. U. Baig, J. A. Khan, and M. F. Khokhar, "Inventory and GLOF susceptibility of glacial lakes in Hunza River Basin, Western Karakorum," Remote Sensing, vol. 13, no. 9, art. 1794, 2021.

[15]	F. Abbas et al., "Glacial lake outburst flood (GLOF) susceptibility mapping in Central Karakoram National Park: A comparative study of machine learning, deep learning, and hybrid models," Natural Hazards (in press) / SSRN Electronic Journal, 2025.

[16]	D. Jiang, S. Li, I. Hajnsek, M. A. Siddique, W. Hong, and Y. Wu, "Glacial lake mapping using remote sensing Geo-Foundation Model," International Journal of Applied Earth Observation and Geoinformation, vol. 136, art. 104371, 2025, doi: 10.1016/j.jag.2025.104371.

[17]	A. Abdrash, "CryoSentinel: A multimodal foundation-model segmenter for glacial lakes in High Mountain Asia from Sentinel-1 SAR, Sentinel-2 optical, and Copernicus DEM imagery," EarthArXiv preprint (not peer reviewed), 2026.

[18]	S. Kaushik et al., "Automated mapping of glacial lakes using multisource remote sensing data and deep convolutional neural network," International Journal of Applied Earth Observation and Geoinformation, vol. 115, art. 103085, 2022.

[19]	J. Thati and S. Ari, "Glacier lakes detection utilizing remote sensing integration with satellite imagery and advanced deep learning method," 2024. [FILL: verify venue, volume and pages]

[20]	L. Yin et al., "Using the improved YOLOv5-Seg network and Sentinel-2 imagery to map glacial lakes in High Mountain Asia," Remote Sensing, vol. 16, no. 12, art. 2057, 2024.

[21]	H. Zhang et al., "An advanced deep learning framework for mapping glacial lakes and its application in the Hindu Kush-Karakoram-Himalaya region," Journal of Hydrology, vol. 664, Part B, 2026.

[22]	P. Adhikari and S. R. Regmi, "Targeted semantic segmentation of Himalayan glacial lakes using time-series SAR: Towards automated GLOF early warning," arXiv preprint arXiv:2512.24117, 2025.

[23]	H. Mustafa et al., "Integrating multisource data and machine learning for supraglacial lake detection: Implications for environmental management and sustainable development goals in high mountainous regions," Journal of Environmental Management, vol. 370, art. 122490, 2024.

[24]	B. Pathak et al., "Automated mapping of glacial lakes in Himachal Pradesh using multi source remote sensing data and machine learning," Scientific Reports, vol. 15, art. 36619, 2025.

[25]	R. Kumar and S. Vijay, "Automated satellite-based glacial lake inventory and change detection in High Mountain Asia," Scientific Reports, vol. 16, art. 5760, 2026, doi: 10.1038/s41598-026-35446-0.

[26]	L. D. Rawlins, C. S. Watson, M. B. Chand, et al., "Glacial Lake Observatory (GLO): Annual dataset of glacial lakes in Nepal and transboundary catchments (2017–2024)," Earth System Science Data, vol. 18, pp. 5143–5165, 2026, doi: 10.5194/essd-18-5143-2026.

[27]	K. He, X. Zhang, S. Ren, and J. Sun, "Deep residual learning for image recognition," in Proc. IEEE CVPR, 2016, pp. 770–778.

[28]	J. Deng, W. Dong, R. Socher, L.-J. Li, K. Li, and L. Fei-Fei, "ImageNet: A large-scale hierarchical image database," in Proc. IEEE CVPR, 2009, pp. 248–255.

[29]	P. Iakubovskii, "Segmentation Models Pytorch," GitHub repository, 2019. [Online]. Available: https://github.com/qubvel/segmentation_models.pytorch

[30]	A. Zupanc, "Improving cloud detection with machine learning," Sentinel Hub Blog (s2cloudless), 2017. [Online]. Available: https://medium.com/sentinel-hub/improving-cloud-detection-with-machine-learning-c09dc5d7cf13

[31]	European Space Agency, "Copernicus DEM — Global and European Digital Elevation Model (COP-DEM)," 2021, doi: 10.5270/ESA-c5d3d65.

[32]	European Space Agency, "Mission ends for Copernicus Sentinel-1B satellite," ESA Applications, Aug. 2022. [Online]. Available: https://www.esa.int

[33]	A. Paszke et al., "PyTorch: An imperative style, high-performance deep learning library," in Advances in Neural Information Processing Systems, vol. 32, 2019.

[34]	D. P. Kingma and J. Ba, "Adam: A method for stochastic optimization," in Proc. ICLR, 2015.

[35]	I. Loshchilov and F. Hutter, "SGDR: Stochastic gradient descent with warm restarts," in Proc. ICLR, 2017.

[36]	F. Milletari, N. Navab, and S.-A. Ahmadi, "V-Net: Fully convolutional neural networks for volumetric medical image segmentation," in Proc. 3DV, 2016, pp. 565–571.